Single Pulley Planned Maintenance Checklist

Daily
• Check pulley body crack or deformation
• Check sheave groove wear condition
• Ensure pulley rotates freely
• Check abnormal noise during rotation
• Check hook or shackle seating condition

Weekly
• Lubricate pulley bearing
• Check pin and bush condition
• Inspect side plate tightness
• Check alignment during lifting

Monthly
• Inspect sheave groove wear measurement
• Check bearing play or looseness
• Inspect hook or shackle wear area

Annually
• Detailed inspection of pulley body and plates
• Check shaft, pin and bearing condition
• Replace worn bush or bearing if required

Every 5 Years
• Major overhaul of pulley assembly
• Replace sheave, pin and bearing
• Load test and certification if applicable

Steel Wire Rope

Daily
• Check broken wire strand
• Check kinks, crushing or bird caging
• Check corrosion or rust condition
• Ensure proper lubrication

Weekly
• Inspect end termination and socket condition
• Check rope diameter reduction
• Check proper winding on drum

Monthly
• Measure rope diameter and record
• Inspect lubrication condition
• Check rope alignment on sheave

Annually
• Detailed visual inspection
• NDT test if required
• Replace if wear limit exceeds standard

Bow Shackle

Daily
• Check body crack or deformation
• Check pin thread condition
• Ensure pin properly tightened

Monthly
• Inspect wear on pin and body hole
• Check corrosion condition

Annually
• Detailed inspection for deformation
• Replace if worn or bent

D Shackle

Daily
• Check crack or bending
• Check pin fit and thread condition

Monthly
• Inspect wear at pin seating area
• Check corrosion or pitting

Annually
• Detailed inspection and load rating marking check
• Replace if deformation found

Safety Pin

Daily
• Check bending or damage
• Ensure proper locking position

Monthly
• Inspect corrosion
• Replace if loose fitting

Chain Link

Daily
• Check crack or deformation
• Check elongation or twist
• Check corrosion

Monthly
• Measure chain elongation
• Inspect weld area of links

Annually
• Detailed inspection of entire chain length
• Replace if wear exceeds safe limit

Steel Wire Rope Sling Planned Maintenance Checklist

Daily Inspection

• Check broken wire strands on sling body
• Check kinks, bird caging or crushing damage
• Check corrosion or rust condition
• Inspect eye loop and thimble condition
• Check ferrule or crimping tightness
• Verify SWL tag or identification marking

Monthly Inspection

• Measure sling diameter reduction
• Inspect lubrication condition
• Check ferrule crack or slip condition
• Inspect thimble seat wear
• Check overall sling length elongation

Quarterly Inspection

• Detailed visual inspection of full length
• Check internal corrosion sign
• Inspect eye splice integrity

Annual Inspection

• Thorough inspection by competent person
• NDT test if required
• Verify SWL marking and certification record

Load Test Interval

• Initial proof load test after purchase
• Periodic load test as per company / statutory rule
• Load test after major repair or ferrule replacement

Standard Rejection Criteria

• Excess broken wire strands
• Severe kink or bird caging
• Ferrule crack or slippage
• Eye loop deformation
• Diameter reduction beyond permissible limit

Portable Oxygen Header Planned Maintenance Checklist

Daily

• Check oxygen cylinder body condition
• Check cylinder valve leakage
• Check header manifold leakage
• Check pressure gauge reading
• Ensure cylinders properly clamped / secured

Weekly

• Inspect header hose pipe condition
• Check regulator leakage and function
• Inspect non return valve / flashback arrestor
• Check manifold isolation valves operation

Monthly

• Check pressure gauge calibration condition
• Inspect header pipeline joints and fittings
• Check cylinder trolley / frame condition

Quarterly

• Leak test of complete header line
• Inspect regulator diaphragm and seals
• Check flashback arrestor internal condition

Annually

• Pressure gauge calibration test
• Regulator servicing / overhaul
• Hydro test validity check of cylinders

Every 5 Years

• Cylinder hydrostatic test (as per statutory rule)
• Replace worn regulators and hoses
• Complete header manifold overhaul